# David Williams — Scan Forwarded 2026-09-16 (Victoria Williams)

Source PDF: `2026-09-16-oil-and-gas-seep-notes-viii-a.pdf` (24 pages, Genius Scan iOS)

Date: 2026-09-16

Author: David Williams

Provenance: scanned by Victoria Williams -> forwarded by David Williams 2026-09-16 20:08 CDT -> JD Longmire

Note: Faithful transcription of David's handwriting (and any printed material in the scan). Transcribed, not paraphrased; shorthand and spellings kept verbatim. Unreadable words marked [illegible]; uncertain readings marked [word?]; struck-through text marked [struck: ...]; marginalia marked [marginalia: ...]. Yellow highlighter passages are rendered in bold; red-ink body text is prefixed [red ink:] or noted inline. The forwarding email gave no description of the contents.

Conventions used below: "mCFPD" = thousand cubic feet per day; "mmCFPD" = million cubic feet per day; "BOPD" = barrels of oil per day; "mta" = million tons per annum; "Tg" = teragrams; "GOM" = Gulf of Mexico; "FC" = Fruitland Coal; "SJB" = San Juan Basin; "CBM" = coal-bed methane; "PF Holly" = Platform Holly; "HS" = hydrostatic; "BHP" = bottom-hole pressure. David's own numbering (①–⑳, ⑳A, ㉑–㉟, S1–S3) is preserved.

## Page 1

# Oil & Gas Seep Notes

[marginalia: red "VIII" and "A)" in left margin; page number "1" top right]

① Oil-Bearing Rocks of the Davenport & Point Reyes areas & Their Implications for Offset along the San Gregorio & Northern San Andreas Faults:

Richard G. Stanley & Paul G. Lillis

In both areas, much of the oil is in oil-bearing ss dikes & sills that intrude siliceous fine-grained strata of the Upper Miocene Santa Cruz Mudstone.

23 oil samples were all derived from Miocene source rocks (Davenport - 6, Point Reyes - 9, Half-moon Bay & La Honda oil fields - 8)

Significant Biodegradation

Oil Migration and ss intrusion happened ~ 9-7 Ma and then the Davenport & Point Reyes areas were separated by 115 ± 10 km of right-lateral displacement along the S.G. & S.A. faults.

Many of the ss intrusions are oil bearing but many are also oil free. Includes dikes & sills with width of few mm to more than 5 m and length of tens of meters.

The ss intrusions formed by injection of fluidized sands from the Santa Margarita ss updip into fractured fine-grained strata of the Santa Cruz Mudstone.

[marginalia: red "" at left] Some of the ss intrusions apparently were hydrocarbon-bearing at the time of emplacement (Phillips 1990; Thompson & others 1999) [red ink:] Early HC Generation**

Some of the ss intrusions were emplaced at very shallow sediment levels and some reached the sea floor and formed extrusives.

[marginalia: red "1/04" at left]

② Natural Oil & Gas Seepage in the Coastal Areas of California: James M. Galloway 1-16-04

Goleta Point: 2 Containment Structures ≈ 4 BCF from 1982-2003 (22 years), 182 mmCF/Year ≈ 500 mCFPD

Famous Seeps: La Brea Tar Pits, Pitch Lake of Trinidad, Perpetual Fires of Baku Azerbaijan.

## Page 2

[marginalia: red "2003" at top left; page number "2" top right]

③ Compositional Changes in Natural Gas Bubble Plumes: Observations from the Coal Oil Point marine hydrocarbon seep field 2003

Jordan F. Clark, Ira Leifer, Libe Washburn

Recent sonar surveys of the South Elwood Anticline seep field indicate that about 1.5 x 10⁵ m³/day of seep gas is emitted from the 3 km² of seafloor, ≈ 5250 mCFPD 1.916 TCF/1000 YRS or 19.16/10000 or 191.6 TCF/100 [marginalia: red, at right: "191.6 TCF / 100,000 YR 19.6 TCF / millions yrs"]

[marginalia: red "11/01" at left]

④ Temporal Variation in Natural Methane Seep Rate Due to Tides, Coal Oil Point area, California

J. R. Boles & J. F. Clark 11-15-2001

2 Large Tents (30m by 30m) Together = 30m x 61m = 1830 m² = 19700 ft² Captures 16,800 m³/day = 594 mCFPD

594 mCFPD / 19700 ft² = 30.15 ft³ / ft² [red ink:] 30.15 ft³/ft²

Rate varies with tides: high tide correlate to reduced flow and low tide with increased flow. Each meter increase in sea height results in 1.5 to 2.2 % decrease in flow.

[marginalia: red "5/00" at left]

⑤ Dissolved Hydrocarbon Flux From Natural Marine Seeps to the Southern California Bight

Jordan F. Clark, Libby Washburn 5-15-2000

Coal Oil Point seep field emits both oil & gas from more than 250 seafloor vents and accounts for 1/2 of all natural seepage along the Northern margin of the Santa Barbara Channel.

Seafloor tents initially collected .4 x 10⁵ m³/day = 1.413 mmCFPD (In 1983) by mid-1990's the rate had declined to .14 x 10⁵ = 495 mCFPD

Using acoustical methods Quigley (1997) and Hornafius (1999) estimated the total emission of natural gas to the atmosphere to be between 1 x 10⁵ and 2 x 10⁵ m³/day or 3,532 to 7,064 mmCFPD,

[red ink:]

1.29 to 2.58 TCF / 1,000 Years

129 to 258 TCF / 100,000 Years

1290 to 2580 TCF / 1 mm Years

12,900 to 25800 TCF / 10 mm Years

129,000 to 258000 TCF / 100 mm Years

## Page 3

[marginalia: red "1999" at top left; page number "3" top right]

⑥ The worlds most spectacular Marine Hydrocarbon Seep (Coal Oil Point, Santa Barbara Channel, California): Quantification of Emissions (1999)

J. Scott Hornafius, Derek Quigley & Bruce P. Lugendyk

50 kHz sonar data was used to estimate natural hydrocarbon emission rates from the 18 Km² marine seep field offshore from Coal Oil Point

Quantitative estimates of seep rates (From Figure 2):

Uncontained { La Goleta seep field = 8.9 x 10⁴ m³/day or 3.1 x 10⁶ CF/day; Area around seep tents = 1.3 x 10⁴ " or 4.7 x 10⁵ " }

Contained man-made seep tents = 1.0 x 10⁴ " or 3.5 x 10⁵ "

Nov 1994 Total 11.2 x 10⁴ m³/day or 3.92 x 10⁶ CFPD

From Abstract & body of report including associated oil emission rate of 1.6 x 10⁴ L/day (100 BPD) The total estimated gas emission rate is 1.7 ± 0.3 x 10⁵ m³/day or 6.0 ± 1.0 x 10⁶ CFPD

[marginalia: red, at left: "Most of present landscape formed in last 25 my or Late Cenozoic Tertiary 50 mm YR"]

Our estimated methane emission rate for the Coal Oil Point seeps (80 ± 12 t/day) is 4 times higher than previous estimates.

[red ink:] * 6.004 mmCFPD = 2.2 BCF/year = 2200 BCF (2.2 TCF)/1000 years = 2200 TCF / million years = 110,000 TCF / 50 million years (Tertiary)

Comparison of 7/95 data with 7/73 data suggests an 80% reduction in seepage rates within 1 km of platform Holly and a 50% reduction within 2 km. This decrease may be due to the reduction in reservoir pressure related to oil production. A reduction in natural seepage rates due to production has also been reported in Texas, Venezuela & Iraq ① Horvitz 1972, ② Landes 1973

① Vegetation & geochemical prospecting for petroleum: AAPG B 56 (1972), p 925-940: L. Horvitz

② Mother nature as an oil polluter: AAPG B 57 (1973) 637-641 K.K. Landes

Hydrocarbon seeps comprise the largest source of marine methane emissions: 8-65 Trillion Grams = 8-65 million Tons Per Year.

Their results for Coal Oil Point seep field of 1600 gm/cm²/yr 4 times the amount Hovland used in estimate above.

⑥a Tracking oil slicks from the coal oil pt. seep field: Ira Leifer AAPG Annual Mtg 2003

18 sq km field emits 105 m³ of gas & 100 BOPD.

## Page 4

[marginalia: red "11/99" at top left; page number "4" top right]

⑦ Decrease in natural marine hydrocarbon seepage near Coal Oil Point, California, associated with offshore oil production. (Nov 1999)

Derek C. Quigley & J. Scott Hornafius

Seepage is most intense at submarine fault conduits and at structural closures along anticline axes.

Seep Tents Rates Initial = 1.050 mmCFPD

Peak = 1.50 mmCFPD because of addition of 1995

1995 = 0.40 " would be 0.750 mmCF without pipe failure

.75 / 1.5 = 50%

Seepage rate is proportional to the ΔP load on deciles[?] laws. 86 Platform Holly production could be the cause. Gas injection at Holly was stopped in 1977 & by 1983 the reservoir pressure fell below hydrostatic and by 1994 the pressure was down 35%. Seep tent emissions began a steep decline in 1990.

This lag of several years can be explained as due to the low K of the Monterey Formation migration pathways.

The seepage area around Platform Holly has decreased by more than 50% in 22 years. Seep tent volumes declined by 50% from 1989 to 1994.

1000 mCFPD / 19700 ft² = .05076 mCFPD / Ft² = 50.76 ft³/day / ft²

[marginalia: red "7/01" at left]

⑧ www.venocoinc.com (State of Ca Website) (7-03-01)

Calif. State Lands Comm. has charted more than 1200 natural seeps in the Santa Barbara Channel. More than 1/2 occur within 3 miles of Coal Oil Point.

Institute for Coastal Studies @ UCSB estimated the oil seepage from Coal Oil Point as 6000 gallons (143 Bbls)/day or 2.3 million Gallons/YR = 54,761 BO/Year = 54.761 million/1000 yrs = 54.761 Billion BO / m Yr (million YRS) = 2.738 Trillion BO / 50 m Yr (Tertiary)

Plus 5 mmCFPD = 1,825 BCFPY = 1.825 TCF/1000 YRS = 182.5 TCF / mYA = 9,1250 TCF / 50 mYA (Tertiary)

GOR = 5000 MCF/143 Bbl = 35 MCF/Bbl

## Page 5

[page number "5" top right]

⑨ Other areas where seeps occur on Continental Shelves

A) Gulf of Mexico (MacDonald 1998)

B) Gulf of California (Simoneit et al 1990)

C) North Sea (Hovland et al 1993)

[marginalia: red "1977" at left]

⑩ Pomona College

1977 State Report identified 2000 leaks along coast of Santa Barbara that year, with another 669 inferred seeps. These amount to 500 BOPD. The area is also rich in natural gas seeps

500 BOPD = 182,500 BO/YR = 182.5 Million BO/1000 YR = = 182.5 Billion BO / mYR = 9.125 Trillion BO / 50 mYA [red underline;] (Tertiary)

[marginalia: red "2001" at left]

⑪ 2001 GSA Mtg Paper 42-0 Abstract: Methane Seepage along faults in the Santa Barbara coastal area, Ca. Geology & modern evidence.

James R. Boles

Tertiary sediments of the Santa Barbara basin are a prolific hydrocarbon source and there's good evidence that the basin has been leaking hydrocarbons, including methane, to the surface for at least 500,000 years.

Over a 10⁵ year time scale, seepage has evolved from high fluids & methane gas up along basin flank faults to college[?] seepage along faults within the basin. Based on U-Th dates from calcite on faults.

[marginalia: red "1972" at left]

⑫ TR 08: California Offshore Oil & Gas Seeps E.R. Wilkinson (1972)

Earliest record was in 1776 noted by Padre Pedro Font near Goleta. Also 1792 by Captain Cook's Navigator in the S.B. Channel.

Areas 1) Point Conception (west of Coal Oil Point)

Magnitude may be as large as Coal Oil Pt

1st documented by Scuba Divers in the 1950's

# of separate vents

Sheet of Tar covering at least 1/4 mi² & forms a 10 to 12 foot scarp. One of 3 localities in So. Calif. Pt. Conception, Coal Oil Pt & Carpinteria

From fractures in dipping Monterey Shale

2) Coal Oil Point (worlds most publicized)

Area of many seeps grouped into 3 main areas: Coal Oil Pt, La Goleta & Isla Vista

## Page 6

[page number "6" top right]

Platform "A" Seep (Caused by a blowout)

At least 1 natural seep in the area from a shallow oil sand in the Repetto formation

Redondo Beach (most active seeps south of Coal Oil Pt)

At least 6 separate seeps along the NW trending Palos Verdes fault (for about 12 miles). Redondo, Manhattan & Venice seeps included

Estimates of flow based on slick size is ≈ 12-15 BOPD per seep & Undetermined gas. Constant stream of gas bubbles great attraction to fish.

Malibu (Caused by Smith[?] quake of 2-9-71)

Limited amount of short duration seeps

⑬ Earth Observatory (Nasa.gov) Media alerts Archive

Roger Mitchell 301-231-0660

2 Times an Exxon Valdez spill worth of oil seeps into the G.O.M. each year according to Roger Mitchell - Earth Satellite Corp. There are over 600 different areas where oil oozes from rocks under the G.O.M.

Calculated by using a thickness of 1/100 th of a mm for the oil film, area covered by surface slicks (from Satellite Data) and est. how long oil stays on surface before it is consumed by bacteria or churned up by waves.

10.9 Million Gallons = 260,000 BO x 2 = 520,000 BOPY (1125 BOA

520 Million BO / 1000 years = 520 BBO / Million Years = 5200 BBO or 5.2 TBO / 10 mYA = 26 TBO / 50 mYA (Tertiary)

[marginalia: red "2003" at left]

⑭ The Fruitland Formation CBM Hydrocarbon System: A Step-by-Step Analysis: W.L. Riese: AAPG 2003 Ann. Mtg. Abstract Only

Basin Subsidence Continued through the end of Cretaceous until Oligocene 40 to 35 mybp, when basin began to take present form.

FC generated gas which filled cleats and was enhanced by meteoric waters. Uplift around 35 mybp allowed additional desorption and along with gas migrated from deeper sources decreased krw sufficiently to prevent further fresh water invasion.

Present-day methane seeps & springs at the outcrop are manifestations that the CBM system is still active.

Fractures which have opened since Rio Grande Rift began to form ≈ 30 mybp, today allow fluids from the FC & deeper formations to come to the surface. This process has been ongoing for hundreds of years (demonstrably) — and have probably been active since the late Miocene.

## Page 7

[marginalia: red "2003" at top left; page number "7" top right]

⑮ Fruitland Formation Coal-Bed-Gas Seeps, No. SJB, Colo.

Agnes F. Fassett: Paper 1-8; Rocky MTN 55th Ann Mtg 2003

Abstract only

Pine River Gas Seep - Gas production from FC CBM wells downdip is unrelated to seep.

[marginalia: red "3/00" at left]

⑯ Coal Bed Methane Development In the No. SJB of Colorado A Brief History & Environmental Observation: BLM SJ Field Office 12/99, Revised 3-15-2000

History of Natural Gas Seeps in the Northern SJB

Historically documented naturally occurring gas seeps throughout the SJB existed prior to oil & gas drilling operations.

Shallow water wells penetrating Fruitland & Menefee coalbeds around the Basin rim have historically produced methane gas.

Known gas seeps include the Carbon Junction area where the Animas River crosses the Fruitland Formation. At this location methane & H2S seeps were commonly recognized as early as the 1930's.

Another well-known site of historic gas seepage is a topographic low in the Hogback Monocline between Valdecil Cyn. & Iron Springs Cyn on the western rim of the SJB.

Other areas of seepage existed at the NE edge of the SJB rim. Ranchers ignited escaping natural gas from water faucets, hollow ponds or reed[?] over streams, or brown soil seeps in entertaining pyrotechnic displays

40 years ago a group of local youngsters who inadvertently cast a campfire ember into the Los Pinos River were duly impressed when the surface of the river ignited in a flash (Hooker, 1999)

As early as 1980-1985, new seeps, not associated with Basin rim outcrops, but interior to the basin, appeared to be forming in pastures in the Animas River Valley south of Durango.

## Page 8

[marginalia: red "1994" at top left; page number "8" top right]

⑰ Methane, CO2, O2, & N2 In Soil Gas Overlying Coal Beds of the Up. Cret. Frt. Form. in the SJB, La Plata Cty, SW Colo.

USGS OFR 94-295: James R. Herring: 1994

Higher rank coal such as Low-Volatile Bituminous & Anthracite, generates ≥ 100 cm³ of methane per gram of coal

Another Report SJB = 230 Billion Tons of FC  1gm = .0022046 pounds

2.3 x 10¹¹

230,000,000,000 Tons x 2000 lbs/Ton x 453.592 grams/lb. x 100 cm³/gram

= 2.0865 x 10¹⁹ cm³ of methane x 1 in³/(2.54)³ cm³ x 1 ft³/(12)³ in³

20.865 x 10¹⁸ cm³ / 2.8317 x 10⁴ cm³/ft³ = 7.3684 x 10¹⁴ ft³ = 737 TCF

[marginalia: red, at left: "A Small Seep, 67 MCF for 30 mm years would fully deplete"]

Seep A, released bubbling gas at 5.3 liters/minute

= 7632 liters/day x .2641721 gals/lt x 1 Bbl/42 gals = 48 BPD

1 Liter = .0062898 Barrels (US Pet)

[marginalia: red "6/01" at left]

⑱ USGS Workshop on a Global Assessment of Geologically Sourced Methane: Tom Lorenson: June 2001

Main objective to account for missing geologically sourced methane in the current global inventory of sources of atmospheric methane. The ancient-carbon for geologically sourced methane sources identified to date account for about 15% of the total methane in the atmosphere, which is 1/2 of the 30% needed to balance the observed atmospheric methane budget.

As a 1st approximation, the observed global rate of methane emission at the seabed was estimated to be about 50 Tg/YR.

Methane emissions from terrestrial sources is believed to be less than from the oceans but knowledge is minimal. Previous estimates from the exploitation of coal & petroleum are 35 to 45 Tg/YR.

50 Tg/YR = 50 x 10¹² gm/Year = 50 x 10¹² gm/YR = 50 x 10⁹ Kg/YR

If 5.8752 Kg/mCF; then = 8.51 x 10⁹ mCF/YR = 8.51 TCF/YR

= 8510 TCF/Year = 8,510,000 TCF/mYA = = 425,500,000 TCF/50 MYA (Tertiary) = 4.255 x 10⁸ TCF / 50 mYA

## Page 9

[marginalia: red "9/99" at top left; page number "9" top right]

⑲ Emission Inventory Guidebook (9-1-99)

Compilation of reports on gas seepages indicates global emissions of 8 to 65 Tg/yr.

Emission factors: 1) highest overall seepage rate is given for the Santa Barbara Channel at 400 gm/yr/m² 2) more + typical factor given for the 60m, North Carolina Coast, & Danish Kattegat, should be used 50/gm/yr/m²

8 to 65 Tg/yr = 1.36 to 11.06 TCF/YR

[marginalia: red, at right: "14 to 111 TCF/10 YR / 140 to 1110 " /100 YR / 1400 to 11,100 " /1000 YR / 14,000-111,000 " /10000 YR / 140,000 to 1,110,000 " / 1 MM YR / 14 to 11.1 " " /50 mMYR"]

[marginalia: red "1993" at left; red, at far left: "Natural Marine Oil Seepage Science (5-24-74) 161184, # 4139"]

⑳ Oil In The Sea III: GESAMP 1993

600,000 tons/year (Wilson et al 1973, 1974)

200,000 tons/year (Kvenvolden & Harbaugh 1983)

[arrow] 200,000 tons/yr to 2,000,000 tons/YR: revised GOM & SBC data applied to Kvenvolden & Harbaugh method

600,000 tons = 180 Million Gallons/42 = 4.286 Million BO

So 1 ton = 7.1433 BO/Ton

4.286 Million BO/yr = 4.286 Billion BO (BBO)/1000 YR = 4286 BBO / mYA = 42,860 BBO / 10 mYA = 2,143,000 BBO or 2143 TBO / 50 mYA (Tertiary)

Range @ 200,000 tons to 2,000,000 Tons/YR = 1,429 to 14.29 Million BOPY = 714 to 7140 TBO / 50 mYA (Tertiary)

GOM = 140,000 Tons/YR, So Cal = 20000 Tons/YR, Alaska = 400 Tons/YR

Total NA = 160,400 Tons/YR

[marginalia: red "1995" at left]

⑳A Oil In The Sea: Inputs, Fates & Effects (1985)

Johnson (1971) & Landes (1973) Compiled by Wilson (1973, 1974)

Compilation of 190 reported submarine seeps can be augmented by 4 more ones 1) Scott Inlet Canada 2) Buchan Gulf, Canada 3) Australian North Coast & 4) Laguna de Tamiahua, Mexico.

## Page 10

[Printed page, not handwritten. Handwritten title at top: "Oil in the Sea: Inputs, Fates & Effects (1985)"; circled "20A" at top right; "47" at right (original page number); page number "10" bottom right.]

FIGURE 2-2 Location of reported marine seeps.

[World map with dot markers showing locations of reported marine seeps; numbered labels "28" (Gulf of Alaska area) and "54" (offshore California) visible.]

SOURCES: Wilson et al. (1973, 1974), Levy (1978), Levy and Ehrhardt (1981), McKirdy and Horvath (1976), Geyer and Giammona (1980), and Harvey et al. (1979).

[Handwritten below figure:] Direct input of PHC from natural sources est. @ 0.025 - 2.5 million metric Tons/yr

Best Est = 0.25 mTA or 250,000 Tons/yr

## Page 11

[Printed photocopied page. Red circled handwritten "20A" at top left; original page number "48". Header: "Oil in the Sea: Inputs, Fates, and Effects (1985) http://www.nap.edu/openbook/0309034795/html/48.html, copyright 1985, 2000 The National Academy of Sciences, all rights reserved"]

TABLE 2-2 Petroleum Resource Estimates

| Source | Amount (Millions of Metric Tons) | Amount (Billions of Barrels) | Reference |

|---|---|---|---|

| Offshore oil "tar" sands | 30,000 | 200 | Weeks (1965) |

| Total offshore petroleum resources | 350,000 | 2,500 | Weeks (1965) |

| Total petroleum resources in place | 1,000,000 | 7,200 | Hunt (1972) |

| Proven oil reserves and potential resources offshore | 14,000 | 100 | Albers et al. (1973) |

| Total proven reserves and potential resources onshore and offshore | 86,000 | 630 | Albers et al. (1973) |

| World exploitable oil (discovered 163,000 mt; undiscovered 141,000 mt) | 304,000 | 2,200 | Halbouty and Moody (1980) |

| Large "tar" deposits | 320,000 | 2,100 | Demaison (1977) |

Geological Implications of Seepage Rates

A comparison of estimated seepage rates with the amount available for seepage can be used to assess the maximum geologic time during which seepages could be sustained.

Table 2-2 lists petroleum resource estimates for several categories of petroleum and illustrates the wide range of resource estimates that have been calculated. Table 2-3 illustrates the comparison. The wide range of assumed seepage rates includes the estimates of Wilson et al. (1973, 1974) but extends downward to 0.02 mta and upward to 10 mta. At the low end, 10,000 mt is near the value of 14,000 mt of total proven reserves and potential resources offshore as estimated by Albers et al. (1973). At the other end of the scale, Wilson et al. (1973, 1974) accepted an estimate of 300,000 mt in place. Because this amount may represent only 1% of the petroleum mobilized from source beds, they assumed that the amount available for seepage may be as much as 30,000,000 mt. This scale available for seepage has been augmented to 100,000,000 mt to attempt to include unknowns with regard to the amount of petroleum that would have been available for seepage during geologic time and will become available in the future during the lifetimes of the seepage.

Table 2-3 shows that to maintain petroleum seepage for a span of geologic time of at least 50 million years (most of the Tertiary period) requires that seepage rates be equal to or less than 2 mta, and

[Handwritten at bottom right: ".02" with a mark]

## Page 12

[Printed photocopied page; original page number "49". Header: "Oil in the Sea: Inputs, Fates, and Effects (1985) http://www.nap.edu/openbook/0309034795/html/49.html, copyright 1985, 2000 The National Academy of Sciences, all rights reserved"]

TABLE 2-3 Maximum Lifetimes (Million Years) of World Oil Deposits

| Oil Available for Seepage (mt) | 0.02 | 0.04 | 0.20 | 0.60 | 1.0 | 2.0 | 6.0 | 10 |

|---|---|---|---|---|---|---|---|---|

| 10,000 | 0.5 | 0.25 | .05 | 0.02 | 0.02 | 0.005 | 0.002 | 0.001 |

| 30,000 | 1.5 | 0.75 | .15 | 0.05 | 0.03 | 0.015 | 0.005 | 0.003 |

| 100,000 | 5 | 2.5 | 0.5 | 0.2 | 0.1 | 0.05 | 0.02 | 0.01 |

| 300,000 | 15 | 7.5 | 1.5 | 0.5 | 0.3 | 0.15 | 0.05 | 0.03 |

| 1,000,000 | 50 | 25 | 5 | 2 | 1 | 0.5 | 0.2 | 0.1 |

| 3,000,000 | 150 | 75 | 15 | 5 | 3 | 1.5 | 0.5 | 0.3 |

| 10,000,000 | 500 | 250 | 50 | 20 | 10 | 5 | 2 | 1 |

| 30,000,000 | 1,500 | 750 | 150 | 50 | 30 | 15 | 5 | 3 |

| 100,000,000 | 5,000 | 2,500 | 500 | 200 | 100 | 50 | 20 | 10 |

[Yellow highlights in the original on: the "0.60" column header; the "10,000,000" row; and the 20 / 50 / 200 cells.]

at the same time the amount available for seepage must be equal to or greater than 1,000,000 mt. If seepage is maintained for 500 million years (most of the Phanerozoic), then seepage rates must be equal to or less than 0.02 mta and the amount available for seepage must be equal to or greater than 10,000,000 mt. The petroleum seepage rate that best seems to accommodate the requirements of reasonable geologic time and reasonable assumptions concerning availability for seepage is 0.2 mta with an uncertainty both upward and downward of an order of magnitude. Thus, the conclusion is reached that the average rate of petroleum seepage over time ranges from 0.02 to 2.0 mta, with a best estimate of 0.2 mta. This value is lower by a factor of 3 than the best estimate of Wilson et al. (1973, 1974) of 0.6 mta.

Erosional Inputs of Petroleum

The amount of petroleum that enters the marine environment by erosional processes has not been estimated before. Previous work by Wilson et al. (1973, 1974) considered only the marine input from natural seeps. Any estimation of erosional input of petroleum into the oceans can only be approximate.

There are at least three places where the erosional input of petroleum into the oceans could be studied in detail. Weaver (1969) showed examples of petroleum seeps at the beach and in the sea cliffs facing the Santa Barbara Channel, where erosion presently is taking place. Giammona (1980) described the Laguna de Tamiahua area where there are onshore and offshore seeps. The Marine Pollution Subcommittee of the British National Committee on Oceanic Research (1980) identified the Dorset coast of southern England as another place where petroleum source rocks as well as petroleum-containing reservoir rocks are exposed. They suggested this area for the study of natural seeps and erosional processes affecting the distribution of petroleum in the marine environment.

Because no direct information is available on erosional inputs of petroleum into the oceans, an indirect approach must be taken. This

## Page 13

[marginalia: red "2002" at top left; red "1 BO = 15 Ltrs" at top right; page number "11" top right]

㉑ Transfer of hydrocarbons from natural seeps to the water column & atmosphere: I.R. Macdonald Geofluids (2002) 2, 95-107

Kvenvolden & Harbaugh (1983): Best estimate of global inputs of 2.5 x 10⁸ L/YR entering the worlds oceans is probably low & should revise upwards since,

Mitchell (1919) calculated that the GOM contributes oil at the rate of .4 to 1.1 x 10⁸ L/YR. [red:] = 252,000 BO/YR / 669,000[?]

The Northern GOM contains over 200 active seeps.

[marginalia: red, at left: "=1.572 MM BO/YR / 1.572 BBO/1000 YR / 1.57 TBO/1 MM YR / 78.5 TBO/50 mMYR"]

㉒ Submarine Seeps: Are they a major source of open ocean oil pollution.

According to Weeks: a max of 700 BBO is available offshore by primary recovery. Since 10% is NGLs: then 630 BBO (~100,000 x 10⁵ m³ oil tons) is the max. amount of likely source for offshore oil seeps.

It has been estimated that offshore seeps amount to 0.5% of annual production. This would imply that submarine seepage would deplete the ultimate reserve of free flowing oil in 20,000 years,

0.5% x 2200 x 10⁶ tons (in 1970) = 11 x 10⁶ tons

*If this amount has been constant since the early Tertiary the amount of seepage would equal 2500 times the free flowing oil & more than 1500 times the total oil in place before offshore production began.

Other Major Seep Locations

S1 Significance of Oil & Gas Seeps in World Oil Exploration Walter K. Link: AAPG V 36 # 8 (8/52)

S2 Levorensen Chapter 2: The occurence of petroleum.

S3 The history of world shallow oil & gas: Chapter 1 Ruth Sheldon Knowles

United States 1) Onshore California, 2) Pennsylvania, & 3) Rocky Mtns

Middle East 1) Iraq (Kirkuk), Iran - most of it.

## Page 14

[page number "12" top right]

Caspian Sea Area; Baku - eternal fire

South America: 1) Venezuela & Trinidad: Guanoco pitch lake, Quirequire seeps, Mene Grande, Lake Maracaibo El Mene, Le Puy

2) Columbia: Infantas & La Cira Pool

3) Peru: de Brea Tar Sands & seeps

Mexico: Tampico Region — thousands of seeps

Canada: Worlds Largest fossil surface seep - Athabasca

Notes from Abstracts of the AAPG Hedberg Conference 9/14-19/2001

① Key Issue Remaining: migration mechanism capable of moving hydrocarbons in a relatively short time (way < 10⁶ years) vertically through thousands of meters of strata w/o observable faults or fractures.

Proposed mechanisms are:

Diffusion: Most likely contributes to near-surface movement but [red underline:] cannot account for relatively rapid movements observed in some studies.

Effusion: requires movement as immiscible fluid through water saturated pore spaces. Would require porous pathway to overcome capillary forces

Advection with moving waters: require significant vertical water movement.

Colloidal Bubble Ascent: microbubbles move vertically via a micro-fracture network.

Continuous Gas Phase Flow: in fractures where small gas bubbles migrate as a continuous non-wetting phase in water-wet rock through microfractures.

⑦ Some Paleoseeps are affiliated w/ancient sedimentary serpentinites that mark dewatering of adjacent ophiolites, tectosomes, or paleo-gas hydrates. Evidenced by concentric, cement-filled conduits, carbonate chimneys, subvertical pipes beneath carbonate mounds.

⑪ Northern GOM: for seep sites surveyed, average total flux of gas (incl as hydrate), extruded mud & donated-biogenic precipitates over the last 3000 years may be expected to be ≈ 3.3 x 10¹⁰ gms/year = 3.3 x 10⁷ Kg/year = 3.638 x 10⁴ tons/yr at 9.07184 x 10⁵ grams/ton

If assume 5.8752 Kg/mCF then = 562 x 10⁶ mCF/YR = 5.62 BCF/YR

## Page 15

[Printed photocopied page. Running head "Oil and Gas Seepages 47". Handwritten circled "S3" at top left. Left edge shows bleed-through fragments of an adjacent page, not transcribed.]

Table 1

Recognition of Oil and Gas Seeps

| Clues | Oil | Gas |

|---|---|---|

| Direct | | |

| Visual | - Oil springs (dark brown to colorless). - Oil lakes or puddles. - Oil drips from rocks. - Rocks or soil are stained brown or black. - Asphalt or tar residue and mounds, often mixed with earth or vegetation (in dry climates, many paraffinic oils evaporate without a trace). - Oil floats on water in puddles, creeks or lakes and forms iridescent films (these can also be formed by iron, but break into fragments when disturbed). - Mud volcanoes (if not solfataric). - Positive ultraviolet or chloroform test. | - Gas bubbles in water or oil. - (Eternal) fires (could be marsh gas or coal gas). - Mud volcanoes and mud springs. |

| Smell | - Oil smell on outcrop or in broken rock fragments. | - Gas smell (esp. the "rotten egg" smell caused by poisonous H2S). |

| Noise | - "Plopping" noise from mud volcanoes or mud springs. | - Gas vents give off a hissing sound. - Gas can explode. - "Plopping" mud in mud volcanoes and springs. |

| Indirect | | |

| Visual | - Tar lumps floating on water or washing up on beach (could be pollution). - Stunted vegetation, bare patches in forest. - Yellowish stain from sulphur; whitish/many-hued stain from bacteria. - Wild or domestic animals coated with oil or tar. - Burnt clays (porcellanites). - Ozarkite, manjak veins, etc. - Saltwater springs and licks (may be seep-connected). | - Stunted vegetation. - Yellowish, whitish or varicolor stain (see oil). - Burnt patches in forest (could be from lighting, etc.). - Salt springs or salt licks (sometimes). - Preferred watering places for animals ("carbonated" H2O). - Accumulations of dead animals, especially in low spots due to poisoning from H2S gas. |

| Other | - Native place names, eg. manjak, [cut off at page bottom] | - Native place names (see oil). |

## Page 16

[Printed photocopied page. Running head "Oil and Gas Seepages 49". Handwritten red at top: "1 Liter = .264 Gals  1 Gal = 3.79 L.A  1 Bbl = 42 Gal = 159.18 Lit"]

Table 2

Some Examples of Direct Production From Oil and Gas Seeps

| Location | Production | Remarks | Reference |

|---|---|---|---|

| Natural flow production | | | |

| Worldwide range | 1 to many 100s liters per day | | Various |

| Tar Springs, California, USA | 2–4 liters/day | | Blumer, 1922 |

| San Timoteo, Venezuela | 50 liters/day | | " |

| Canyon City, Colorado, USA | 75 liters/day | | " |

| Sulphur Mt., California, USA | 150 liters/day | | " |

| Coal Oil Point, California, USA | est. 8,000 to 140,000 liters/day [red:] 50.26 to 879.5 BPD | Offshore | Wilson, 1973 / Mikolaj, 1972 |

| Santa Monica, California, USA | 320–3,200 liters/day [red, struck number above:] 2 to 20 BPD | Offshore | Mikolaj, 1972 |

| Oil Creek, Penna., USA | 10–20 liters/day | | |

| Ahwaz, Iran | 80–100 liters/day | | Coomber, 1938 |

| Maidan-i-Naphtun, Iran | 3,000 liters/day [red:] 18.9 BPD | | Cunningham-Craig, 1920 |

| Estimate of worldwide marine seepage | 640,000 to 16-million liters/day [red:] 4021 to 100515 BPD / "Best" estimate is 1.6-million liters/day | | Wilson, 1973 |

| Umiat, Alaska, USA | 800 liters/day | | Johnson, 1971 |

| Shelikoff Strait Alaska, USA | 100 liters/day | Offshore | Johnson, 1971 |

## Page 17

[Printed photocopied page. Running head "Shallow Oil and Gas Resources 50". Handwritten circled "S3" at top right.]

Table 2

continued

| Location | Production | Remarks | Reference |

|---|---|---|---|

| La Brea Pitch Lake, Trinidad | 100–150 tons/day | Estimated possible influx | Author |

| Seep production from hand-dug pits or shallow wells | | | |

| Ghana | 800–1,200 liters/day | Wells | Khan, 1970 |

| Barbados | 700 BOPD and 1 MM cu ft gas per day | ± 20 wells | Bull AAPG Annual Review, 1983 |

| Dominican Republic | "several hundred BOPD" | Shallow wells Information not verifiable, not sustained | Vaughan, 1921 |

| Guyana | 140,000 cu ft gas per year | Water well | Kugler 1942 |

| Burma | 32,000 to 64,000 liters/day [red:] 201 to 402 BPD | ± 500 pits | Tainsh 1950 |

| Dagah Shabell Somali Republic | 10 liters/day | Pit | Beeby-Thompson 1918 |

| Production from seeps using moderate to sophisticated technology | | | |

| Athabasca, Canada | 160,000 to 170,000 BOPD | Sophisticated high cost mining, upgrading and transportation | |

[Bottom row cut off; continues on next page.]

## Page 18

[Printed photocopied page. Running head "Oil and Gas Seepages 51". Continuation of Table 2.]

| Location | Production | Remarks | Reference |

|---|---|---|---|

| Pitch Lake, La Brea, Trinidad | 11.3 million tons from 1893–1980 | Mostly manual mining, but mechanized and relatively sophisticated upgrading and shipping | Various |

| Selenizza, Albania | 11,000 tons per year | 1963 figure | Various |

| Val de Travers Switzerland | 15,000 to 50,000 tons/year | Production ceased | Various |

| Teca and Cocorna, Colombia | ± 5,000 BOPD | Production from 50 wells on or near seep (± 100 BOPD on pump) and using steam stimulation | The Texaco Star, 1984 |

| Austria | < 100,000 Bbls cumulative | No production now | |

| France | < 300,000 Bbls cumulative | No production now | |

| West Germany | 300,000 Bbls cumulative | No production now | |

| Thailand (Mae Fang) | ± 100,000 Bbls cumulative; individual wells avg. 10 BOPD appr. End 1958 est. 350–500 BOPD from 15 wells | Government made major efforts to develop this seep area between 1930–1960 | Various |

| Japan (Kanto) | Approx. 300-million m³ gas p.a. | In shallow wells | Various |

## Page 19

[page number "13" top right]

⑫ 25 Miles of Fruitland surveyed revealed strong seeps (above hazard levels) along 7 miles of O.C.

⑬ Rate of Migration: max during mud volcanoe eruption 10⁻² m/s; min microseepage 10⁻¹⁰ m/s

⑮ Onshore Calif? Pico Formation; Shallow gas identical to production gases from field 4.5 mi. SE which has produced > 3 BCF of dry gas (Pico Form.)

Measured seepage rates range upwards from about 1 mL/min to as much as 9 liters/minute of gas.

⑯ Total net input to atmosphere at about 540 Tg (teragrams = 10¹² g of methane) 1,000,000,000,000 Same as Trillion

1st Approx of global rate of seabed emissions estimated at about 50 Tg/YR

⑰ In 1975, the estimated rate of natural seepage of oil into the marine environment ranged widely from 200,000 to 6,000,000 mt/a with a best est. of 600,000 mta. Based on extensive geological considerations but used extrapolation from only a few highly suspect seeps. In 1985, estimates of individual oil seep rates had not changed. A revised estimate was calculated based on the amount of oil present that could seep over reasonable periods of geologic time. The new estimate ranged between 20,000 to 2,000,000 mta with a best est. of 200,000 mta.

Current studies using new technologies calculate at;

GOM = 150,000 mta

Calif = 20,000 "

offshore Alaska = 20,000 "

190,000 m Just Part of North America

⑲ Canada "The Hot Pot": These seepages are inferred to represent the discharge of regional gas migration systems of 10's to 100's of kilometers.

㉑ Coal Oil Point 100,000 cm of gas & 100 BOPD: If the methane added to the atmosphere from exposed seeps would be double the amount now estimated to result from marine seeps to 20 - 100 trillion grams (Tg) per year.

## Page 20

[page number "14" top right]

㉔ We can potentially estimate the flux rate of hydrocarbons to the surface by using microbially based calculations to determine how much gas per unit of time is required to support the level of Hydrocarbon Degrading culture present in Soil.

Need whole article

㉜ Surface hydrocarbon seeps & soil geochemical anomalies have been shown to appear & disappear in relatively short times (weeks to months to years)

Horvitz (1969) documented the disappearance in < 20 years of a soil hydrocarbon anomaly above Hastings Field, Brazoria Cty, TX.

Sivaborvun (1974) during Soc. Roc., a surface geochem anomaly increased in intensity over Hilbig field in Bastrop Cty, TX.

Coleman & others (1977) demonstrated leakage of gas from 1000-meter deep gas storage reservoirs into shallow water wells within a 1 year period

Jones & Burtell (1996) summarized results for a # of studies of Hyd flux variations in natural seeps as well as underground storage reservoirs

Hitzman & others (2001) & Rice & others (2001) - documented sign. changes in microseepage int. & distribution over existing fields in in < 1 year

Jones & Burtell (1996) - repeat soil gas concentrations above gas storage reservoirs respond within 2-4 days to press changes in the reservoir

Based on these & other studies, the rate of hydrocarbon migration & microseepage varies from less than 1 meter to tens of meters per day (Arp 1992, Matthews 1996, Klusman & Saeed 1996, Clayton & Dando 1996, Brown 2000, Price 2001).

Not limited to microseepage

Whelan & others (1994) - documented new O&G entering some of the reservoirs in Eugene Island Block 330 in the GOM, off La. Significant Δ in oil composition in 3-10 years.

Nunn & Meulbroek (in press) - GOM data, make strong case for 2-10 km of vertical migration of brine & assoc. hyd. on time scale of 1-2 million yrs.

㉞ Theories of hydrocarbon microseepage need to account for the robust nature of vertical migration even in areas not expected to include significant vertical fracturing to the surface.

㉟ This suggests that alkanes present as free gas within the soil had a residence time of only a few days.

## Page 21

[Printed page. Red handwritten at top left: circled "32" and "need to order several papers"]

AAPG HEDBERG CONFERENCE

"Near-Surface Hydrocarbon Migration: Mechanisms and Seepage Rates"

SEPTEMBER 16-19, 2001, VANCOUVER, BC, CANADA

The Dynamic Nature of Hydrocarbon Microseepage: An Overview

By

Dietmar Schumacher, Geo-Microbial Technologies, Inc.

Hydrocarbon microseepage rates and near-surface hydrocarbon concentrations can vary significantly over time. Surface hydrocarbon seeps and soil geochemical anomalies have been shown to appear and disappear in relatively short times, weeks to months to years. Horvitz (1969) documented the disappearance in less than 20 years of a soil hydrocarbon anomaly from above Hastings field in Brazoria County, Texas. Sivaborvon (1974) reported that a surface geochemical anomaly increased in intensity over Hibig field in Bastrop County, Texas, during repressurization for secondary recovery studies. Coleman and others (1977) demonstrated leakage of gas from 1000-meter deep gas storage reservoirs into shallow water wells within a one year period. Jones and Burtell (1996) summarized results for a number of studies of hydrocarbon flux variations in natural seeps as well as from underground storage reservoirs. Hitzman and others (2001) and Rice and others (2001) have both documented significant changes in microseepage intensity and distribution over existing fields in time intervals as short as one year.

These studies clearly demonstrate rapid variations in gas flux associated with pressure changes in gas storage caverns and other oil and gas reservoirs. For example, Jones and Burtell (1996) report that soil gas hydrocarbon concentrations above gas storage reservoirs respond within 2-4 days to pressure changes in the reservoir or storage cavern. Similarly, soil gas concentrations above an underground coal gasification reactor changed dramatically within days of significant changes in the status of the coal-burn. Based on these and other studies, the rate of hydrocarbon migration and microseepage varies from less than one meter per day to tens of meters per day (Arp, 1992; Matthews, 1996; Klusman and Saeed, 1996; Clayton and Dando, 1996; Brown, 2000; Rice and others, 2001).

Data supporting the dynamic nature of hydrocarbon migration is not limited to microseepage. Whelan and others (1994) have documented that new oil and gas are entering some of the reservoirs in Eugene Island Block 330 field in the Gulf of Mexico, offshore Louisiana. Significant changes in oil compositions were observed to be occurring over three to ten years. Furthermore, the oil compositions show that the new oil is migrating into the reservoir from deeper, hotter formations and is not just the result of localized production-related lateral migration. Nunn and Meulboeck (in press), also working with Gulf of Mixico data, make a strong case for 2-10 km of vertical migration of brine and associated hydrocarbons on time scales of 1-2 million years.

These observations coupled with those previously reported for near-surface microseepage, demonstrate quite clearly that hydrocarbon migration and hydrocarbon microseepage are dynamic processes, processes leading to measurable changes on a time scale of months to years.

## Page 22

[Printed page. "References Cited". Red handwritten marks in left margin: "O" by Arp and Brown refs, "HAVE" by Clayton, "Have" by Klusman, "HAVE" by Matthews, red "**" by Whelan.]

References Cited

Arp, G.K., 1992, An integrated interpretation for the origin of the Patrick Draw oil field sage anomaly: AAPG Bulletin, v. 76, p. 301-306.

Brown, A., 2000, Evaluation of possible gas microseepage mechanisms: AAPG Bulletin, v. 84, p. 1775-1789.

Clayton, C.J., and P.R. Dando, 1996, Comparison of seepage and seal leakage rates, in D. Schumaker and M. Abrams, Hyudrocarbon migration and its near-surface expression: AAPG Memoir 66, p. 169-171.

Coleman, D.D., W.F. Meents, C.-L. Liu, and R.A. Keough, 1977, Isotopic identification of leakage gas from underground storage reservoirs: a progress report: Illinois State Geological Survey, Illinois Petroleum, no. 111, p. 10.

Hitzman, D.C., B.A. Rountree, J.D. Tucker, and S. Smith, 2002, Integrated microbial and 3-D seismic surveys discover Park Springs (Conglomerate) field and track microseepage reduction, in Surface Exploration Case Histories: Applications of geochemistry, magnetics, and remote sensing: AAPG Studies in Geology no. 48 / SEG Geophysical References Series no. 11, p. 59-65.

Horvitz, L., 1969, Hydrocarbon prospecting after thirty years, in W.B. Heroy, ed., Unconventional methods in exploration for petroleum and natural gas: Dallas, Texas, Southern Methodist University Press, p. 205-218.

Klusman, R.W., and M.A. Saeed, 1996, Comparison of light hydrocarbon microseepage mechanisms, in Hydrocarbon migration and its near-surface expression: AAPG Memoir 66, p. 157-168.

Matthews, M.D., 1996, Migration – a view from the top, in Hydrocarbon migration and its near-surface expression: AAPG Memoir 66, p. 139-155.

Rice, G.K., J.Q. Belt, Jr., and G.E. Berg, 2002, Hydrocarbon soil gas pattern changes during a West Texas waterflood, in Surface Exploration Case Histories: Applications of geochemistry, magnetics, and remote sensing: AAPG Studies in Geology no. 48 / SEG Geophysical References Series no. 11, p. 157-174.

Sivaborvon, V., 1974, Re-study of hydrocarbon distribution around Hilbig oil field, Bastrop County, Texas: Master's thesis, University of Texas, Austin, Texas, 79 p.

Whelan, J.K., M.C. Kennicutt, J.M. Brooks, D. Schumacher and L.B. Eglinton, 1994, Organic geochemical indicators of dynamic fluid flow processes in petroleum basins: Organic Geochemistry, v. 22, p. 587-615.

## Page 23

[page number "15" top right]

South Ellwood Field Data

Reservoir Size = 1.2 BBO  Expansion Area = 1.0 BBO

Cum (1966-2000) Oil = 60 mm BO   EUR = 69.4 MMBO

Gas = 40 BCF    EUR = 46 BCF

Monterey Only

Cum (1972 - 5/2006)  Oil = 57.5 MMBO      Peak Prod = 1984

Gas = 49.4 BBO       3.45 mm BOPY

WTR = 63.0 mm BW     2.95 BCFPY

| Formation | BH Orig PSI/Ft | BHT | GOR | H2S[?] |

|---|---|---|---|---|

| Sisquoc | 615 | .455 | 99 | — | 10 |

| Monterey | 1600 | .478 | 150 | 500 | 500 |

| Rincon | 2205 | .411 | 190 | 346-1000 | 150 |

| Vaqueros | 3650 | .619 | 208 | 3947-4800 | 60 |

| Sespe | 3650 | .608 | 208 | 3947-4800 | 150 |

① 1996 (AAPG ABST): Hornafius

→ Substantial Reduction In Natural Seepage w/I 1 mile of PF Holly

Since production from Monterey began In 1972

Monterey is source for gas

60% decline in Seep Gas Captured by Tents (1 mi. East)

Corresponds to 40% reduction in BHP under PF Holly & 43 mm BO & ≈ Same volume of water since 1972

② 12/2003 (AGU): Boles

initial BHP 5% over hydrostatic. Currently 25% under HS.

③ 9-15-99 (JGR): Hornafius main ⑥

18 km² marine seep field off Coal Oil Pt includes seep tent

gas emission rate = 1.7 x 10⁵ m³/day = 6.004 mmCFPD = 2,192 BCF/year includes seep tent = .19 x 10⁵ " = 0.671 " = 0.25 "

oil seep rate = 1.6 x 10⁴ L/Day = 100 BOPD = 36,500 BO/year

[marginalia: at left, in a column: "Tertiary 50 mAYR / 1.1 Million TCF / 125000 TCF / 18.25 Trillion BO"]

| | Year | 100Y | 1000Y | 10KY | 100KY | 1 MM | 10 MM |

|---|---|---|---|---|---|---|---|

| Total GAS | 2.192 BCF | 219.2 BCF | 2,192 TCF | 21.92 TCF | 219.2 TCF | 2192 TCF | 21,920 TCF |

| Tent GAS | 6.25 BCF | 25 BCF | 250 BCF | 2.5 TCF | 25 TCF | 250 TCF | 2500 TCF |

| Total Oil | 36500 BO | 3.65 MMBO | 36.5 MMBO | 365 MM | 3.65 BBO | 36.5 BBO | 365 BBO |

④ 11/99 (Geology): Quigley Main ⑦

Nearshore Seeps (@ Coal Oil Point): Oil seepage directly from the outcrop of the Monterey Formation exposal in the axis of the Coal Oil Point Anticline.

Farther offshore seepage passes through the overlying Sisquoc Formation Cap rock and includes both oil & gas. The offshore gaseous seepage is controlled by the local geologic structure which trends west-northwest. Seepage is most intense at submarine fault conduits and at structural closures along anticlinc axis.

## Page 24

[page number "16" top right]

Seep Tents were installed in 1983 ≈ 1 mile SE (Updip) from Platform Holly & the South Ellwood Field. The collection rate was approximately 1000 MCFPD. In 1986 Flaps installed on the tents increased the rate to ≈ 1600 MCFPD.

Production From the Monterey began @ South Ellwood in 1972. Gas was reinjected in the field til 1977. By 1983 pressures in the Monterey began to decline below Hydrostatic. By 1994 the total pressure drop was ≈ 35%.

The seep rate declined rapidly in 1989, Approximately 6 years After press decline started, by 1994 the rate was only 400 to 500 MCFPD. Pressures in 1994 7.54 MPA/11.9 MPA = % of orig. 1.5 km From Platform under seep tents 9.65/11.9 MPA = % of orig.

Seepage area has declind by more than 50%.

⑤ 11-15-2001 (JGR): Boles & Clark MAIN ④

Measured Seep Rates

1982: 9 x 10⁵ m³/mo x 35.32 = 317.88 x 10⁵ ft³/mo = 31,788,000 CFPM / 30.4 = 1045 MCFPD

[marginalia: at left: "All Seep Tent Starts[?]"]

1986: + 3 x 10⁵ m³/mo x 35.32 = 105.96 x 10⁵ ft³/mo = +10,596,000 CFPM / 30.4 = + 349 MCFPD

Total = 1045 + 349 = 1354 MCFPD

Between 1986-1989: 1.4 x 10⁶ m³/mo x 35.32 = 49.448 x 10⁶ ft³/mo Peak Rate = 49,448,000 CFPM / 30.4 = 1627 MCFPD

Between 1990-1994: 3 x 10⁵ m³/mo = 349 MCFPD

Dropped To 1999: 5 x 10⁵ m³/mo = 582 MCFPD

[marginalia: at left: "Gradual Inc"]