# David Williams — VIII-C Diffusion Models and Rates

Source PDF: `2026-09-23-viii-c-diffusion-models-and-rates.pdf` (32 pages, Genius Scan)

Date: 2026-09-23

Author: David Williams

Note: Faithful transcription of David's handwriting. Pages 1–10 of this scan package repeat the VIII-B "Hydrocarbon Microseepage & Geochemical Exploration" material already fully transcribed in `2026-09-23-viii-b-hydrocarbon-microseepage.md`; they are summarized structurally below rather than re-transcribed. Pages 11–32 (David's own pagination 1–22) are the VIII-C "Diffusion Models & Rates" notes, transcribed in full. Transcribed, not paraphrased; shorthand and spellings kept verbatim. Yellow-highlighted passages rendered in bold. Unreadable words marked [illegible]; struck-through text marked [struck: ...]; marginalia marked [marginalia: ...].

## Pages 1–10 (structural note — content duplicates the VIII-B scan package)

These pages repeat the VIII-B "Hydrocarbon Microseepage & Geochemical Exploration" notes (see `2026-09-23-viii-b-hydrocarbon-microseepage.md` for the full transcription):

Page 1: VIII) B) Hydrocarbon Microseepage & Geochemical Exploration. ① "Finding New Pays in Old Plays: Recent Geochemical-Exploration Successes from Texas & OK: Schumacher: OGS [citation] 107 (2002)". Microseepage Changes Over Time: Park Springs Conglomerate Field, Montague Co., Texas; SE Vassar Vertz Sand Unit, Payne County, Okla. [marginalia: red "2002" at left; red "5/01" at left]. ② "Soil Gas Survey Detects Microseepage Through Thick Evaporite Sequence In Egypt: El-Bishlawy: OGJS 7-01".

Pages 2–5: Continuation of the VIII-B outline notes (microseepage literature notes: Tedesco/Jedesen rapid seep rates, iodine vs. soil-gas surveys, Dolley/Keota/Leyden fields).

Page 6: "Luca Technologies" handwritten note (biogenic methane production in coal fields; 30 samples from 20 HC reservoirs).

Pages 7–10: Printed Luca Technologies press release, embargoed November 16, 2004 — "New Discovery by Luca Technologies of Real-time Natural Gas Formation in the Powder River Basin Offers Prospect for Renewable Energy Resource for Generations to Come."

## Page 11 (David's p. 1)

Diffusion Models & Rates

[I-A] Migration, Accumulation & Retention of Petroleum In The Earth [marginalia: red "VIII C)" at left]

→ Petroleum accumulations are seldom found at the site of deposition of the source material and subsequent genesis. In fact, migration out of the source rock & accumulation in a trap is essential to formation of a petroleum accumulation of sufficient size to be of economic interest. Just as this mobility leads to accumulation, it also leads to leakage out of the reservoir, either to another trap or to the surface.

→ The major fluid in compacting fine grained rocks such as shales is water. The fluid pressure history in such rocks is dominated by water and by the rate at which it can flow out of the rock. Water flow & pressure gradients can serve to transport petroleum from source rock to reservoir & under favorable conditions contribute to preservation of the accumulation.

• Losses of Petroleum From Reservoirs Over Geologic Time.

→ Seepage of Petroleum to the surface is a common occurrence in many petroliferous basins. A variety of micro methods have been developed for detection of petroleum HC's in surface soils & more recently in water offshore as an aid to exploration.

→ The slowest mechanism for transport is diffusion except when the reservoir is overlain by rocks overpressured relative to the reservoir.

→ The loss of light hydrocarbons from reservoirs by diffusion to the surface of the earth is a major mechanism for depleting petroleum accumulations over geologic time.

→ Ficks Law describes the diffusion of gas components, through the pore waters of the rock overlying an accumulation.

∂Ci/∂t = Di ∂²Ci/∂z²

→ Calculated diffusion rates of 1 to 8 cm/1000 years. [red "WAY wrong" at right]

## Page 12 (David's p. 2)

[1] Diffusion of Fluids Through Porous Media with Implications in Petroleum Geology: G.N. Pandey, M. Rosin Tek & Donald L Katz.

• Diffusion of Helium Through Caprock

Core diffusion coefficient = 4.1 x 10⁻⁶ cm²/sec

Caprock - 400' Thick assume water filled

75% Helium - 27% N2 @ 740 psia

11000 AC Storage Field

He diffusion = 2.78 x 10⁵ ft³ = 278 MCF in 50 years

He diffusion = 8 BCF in 1 million yrs

→ The larger loss in 1 million years raises the ? of how He has been retained in natural gases.

[2] Diffusion of light hydrocarbons through near-surface rocks: D. Leythaeuser, R.G. Schaefer

→ The concentration-depth trends computed by the mathematical model based on Fick's Law closely match those measured for both case holes in west Greenland. An assumption of time available for diffusion in these cases makes it possible to determine absolute diffusion rates of HC through a column of rocks in natural conditions. These diffusion coefficients vary between 10⁻⁶ & 10⁻⁸ cm²/sec for ethane to heptane range HC's & decrease sharply with molecular size.

[3] Near-Surface Evidence of Migration of Natural Gas from Deep Reservoirs and Source Rocks: W. Stahl, E. Faber

→ Examples presented show that the isotopic composition of the methane adsorbed in near-surface sediments above deep HC reservoirs is similar to the isotope ratios of the methane from the deep reservoir. This similarity is evidence that migration occurs from the deep HC reservoir & source rocks to the surface.

## Page 13 (David's p. 3)

[4] (DL-1) 66-4(4-82) 408-429: Role of Diffusion in Primary Migration of Hydrocarbons: Leythaeuser, Schaefer, Yukla

→ Based on deterministic model

• It was shown that diffusion represents an effective process for primary migration of gas but not for oil.

• For example, a cumulative amount of 10⁹ kg of methane (1.5 x 10⁹ std m³ or 5.3 x 10¹⁰ scf = 53 BCF) has escaped by diffusion in 540,000 years from a certain volume (1000 km² by 200m thick) of a high-mature gas-prone Mesozoic source rock in western Canada.

• The origin of HC accumulations with high gas-to-oil ratios in low-mature sediments in geologically young basins ("early gases & condensates") can be explained by an early phase of primary migration predominantly based on diffusion.

• Compositional trends among reservoir gases of several multiple-pay gas fields in Louisiana represent evidence for diffusive transport of the HC's from a uniform source rock at depth to their present accumulation sites. In the shallow pay zones, compounds of high diffusivity are enriched. For example, in the Sligo gas field of La. the methane/ethane and the iso-butane/N-butane ratios increase from 10.2 to 36.0 and from 0.86 to 1.13, respectively, from the deepest to the shallowest of the five productive reservoir sands, which are spread over a depth interval of 5500 ft.

• Harlingen field, Holland, by diffusive loss through 400m of shale cap rock, the initial amount of methane in place of 1.93 x 10⁹ m³ (6.8 x 10¹⁰ SCF = 68 BCF) is reduced by one-half over a period of 4.5 Million Years. This leads us to propose the concept that large gas accumulations can persist through extended periods of geologic time only as dynamic systems reaching some kind of steady-state equilibrium between diffusive loss through the caprock & continuous replenishment from the source rock. \*

• Many petroleum geologists & geochemists seem to agree that there is probably no single mechanism for HC migration. Instead, it is likely that [struck: the] several different mechanisms are operating at different times and stages of burial.

• Assessment of the mass transport rate of HC's in geologic environments by diffusion processes has been a matter of dispute.

## Page 14 (David's p. 4)

→ Smith (1971), calculated that 140 Million Years are required for the diffusion of methane through a column of 1740m of clastic sediments of 15.7% mean porosity. However, Pandey et al (1974) calculated that within 50 years 7870 m³ of helium escapes from a reservoir accumulation through 400 ft (122m) of limestone cap rock.

→ Much of the previous controversy appears to result from inadequacies with respect to the selection of diffusion coefficients for individual hydrocarbons & with respect to the assumptions made for any model calculations. Most were based on experimentally determined diffusion coefficients

→ For this study: Quantification of the light-hydrocarbon data for two west Greenland core holes permitted calculation of absolute diffusion rates for ethane to heptane range molecules (Leythaeuser 1980). This calculation was based on Fick's law of diffusion and on the assumption of the surface exposure for these Upper Cretaceous shales. This time period was considered to be equal to the age-dated retreat of the ice cap from coastal west Greenland.

\* This is why calculated coefficients are too low and give too long migration times - compared to observed rates from Geochem studies. If they used

12000 Years / 4000 Years = 3 times Too Slow

40,000 Years / 4000 Years = 10 " " "

120,000 Years / 4000 Years = 30 " " "

→ Case histories of compositional evidence for diffusive transport of light HC's, based on reservoir-gas analysis, are most likely to occur in gas fields of multiple pay zones in geologically young and actively subsiding basins.

3 Examples from Searching (Moffit and Plunkett, 1976)

Sligo gas field - Bossier Parish - 5 Gas SS over interval of 5500'

Washington " - St. Landry " - 2 strat Adjacent SS, 150 ft Apart

Rayne " - Acadia " - 5 SS interval in 2800 ft thick Shale/Silt

In all 3 fields, the variation in gas composition with reservoir depth (Table 2) is consistent with the assumption of increasing distances

## Page 15 (David's p. 5)

of diffusive transport from a uniform source rock at depth to their present accumulation sites. This increasing migration distance has resulted in compositional fractionation effects, similar to the ones predicted on the basis of our diffusion model.

→ In the subsurface, diffusion of light hydrocarbons can occur also through the cap rocks of HC accumulations (Thompson, 1979)

∴ can calculate the time it takes to destroy a gas field by using the diffusion coefficients from the source rock studies & applying it to the gas field cap rocks.

Harlingen gas field in Holland (IGIP = 1.93 x 10⁹ std m³ or 6.8 x 10¹⁰ scf = 680 BCF) (400 m thick cap Rock)

In 4.5 MY would loose 50% of GIP

400 m Cap Rock takes 4.5 Million Years

300 m " " " 2.7 " "

200 m " " " 1.3 " "

100 m " " " 0.4 " "

\* Existing gas accumulations can be destroyed in rather short periods of geologic time! \*

→ If this is true, known gas fields in older strata like lower Paleozoic Reservoirs can be explained only by assumption of a relatively young accumulation age or by the assumption of a much longer duration of the hydrocarbon generation process than currently accepted.

\* Table 2. Composition of Natural Gases in Three Multiple-Pay Fields in Louisiana\* [printed]

| Field | Reservoir | Depth, ft (m) | C₁/C₂ | C₂/C₃ | n-C₆/n-C₅ | i-C₄/n-C₄ |

|---|---|---|---|---|---|---|

| Rayne | Klumpp A sand | 10,700 (3,261) | 21.0 | 2.81 | 0.61 | 1.26 |

| | Klumpp D sand | 11,200 (3,414) | 19.2 | 3.02 | 0.61 | 1.15 |

| | Homeseekers D₂ sand | 12,300 (3,749) | 17.2 | 3.05 | 0.59 | 1.07 |

| | Homeseekers E sand | 12,600 (3,840) | 16.9 | 2.47 | 0.39 | 1.09 |

| | Nodosaria A sand | 13,500 (4,114) | 14.2 | 2.05 | 0.65 | 1.06 |

| Washington | Cockfield B sand | 9,150 (2,789) | 18.0 | 0.72 | — | 1.06 |

| | Cockfield D sand | 9,300 (2,834) | 14.8 | 1.29 | — | 0.78 |

| Sligo\*\* | Paluxy Fm. Schuler sand | 2,540 (775) | 36.0 | 3.76 | 1.50 | 1.13 |

| | Rodessa Fm. "Jeter zone" | 4,030 (1,230) | 27.6 | 3.93 | 1.33 | 0.59 |

| | Sligo Fm. Pettet ls. | 4,810 (1,415) | 27.0 | 3.97 | 2.00 | 1.26 |

| | Hosston Fm. | 5,140 (1,540) | 22.4 | 4.10 | — | 1.00 |

| | Cotton Valley Grp. "D" sand | 8,030 (2,450) | 10.2 | 2.65 | — | 0.86 |

\*Based on Lafayette and New Orleans Geological Societies (1968).

\*\*Approximate depths in Sligo field were obtained from cross section (Lafayette and New Orleans Geological Societies, 1968).

## Page 16 (David's p. 6)

In this since, gas accumulations could be considered to persist through extended periods of geologic time only as dynamic systems reaching some kind of steady-state equilibrium between loss through the caprock and continuous replenishment from the source rock.

Summary and Conclusions

Molecular diffusion through the water-saturated pore space of source rock - type shales represents an effective process for primary migration of gas. Our model calculations have shown that this process by itself can account for transportation of such quantities of gas with geologic time which are in the order of commercial size fields.

Due to an exponential decrease of the diffusion coefficient with increasing carbon number, the diffusive flux rate of petroleum-range HC's is so low that this mechanism probably does not contribute significantly to primary migration of petroleum-range HCs.

Where diffusion is the predominant mechanism of primary migration of gas, the composition of the gas accumulating initially in an adjacent reservoir trap is significantly different from that generated in the source rock.

This mechanism offers a viable explanation for the origin of HC accumulations with high gas-to-oil ratios and also concentrates which have been discovered in immature to low-mature sediment in many young basins worldwide (so called "early condensates & gases").

[DL-2] 67-6(6/83) 889-895: Diffusion of Light HC's in Subsurface Sedimentary Rocks: Leythaeuser, Schaefer, Pooch

Individual molecular species are ranked in sequence of increasing effective diffusion coefficients. Benzene & Toluene appear to have anomalously high transportation rates.

Concentration depth plots reveal redistribution of light hydrocarbons.

## Page 17 (David's p. 7)

[6] 67-6(6/83) p932-952: Predictions of Oil or Gas Potential by Near-Surface Geochemistry: V.T. Jones, R.J. Drozd

Discussion

The exceedingly large hydrocarbon seeps that must have existed for a very long time over the Pineview Field imply a fair loss of hc's from the reservoir. Perhaps these seeps even imply that primary migration is still ongoing, from the Cretaceous sediments beneath the Pineview field.

Conclusion

We have demonstrated that a well ordered chemical relationship among light hc gases in near-surface soils & gases exists as noted by Nikonov (1971) in reservoirs and by Pixler (1969) in gas shows from drilling wells.

[7] JPT 1984, p132-140: Leroy Storage Facility, Uinta County, Wyoming: A Case History of Attempted Gas-Migration Control.

→ The total cumulative amount of migration over 130 Months of storage history appeared to be about 600 MMCF (17 x 10⁶ m³).

[red: 600,000 MCF / 3954 days = 152 MCFPD]

[8] DL-3/68-2 (2/84) p196-219: A Novel Approach for Recognition & Quantification of HC Migration Effects in Shale-Sandstone Sequences: Leythaeuser, MacKenzie

Several migration phenomena documented in the present study, do not, at first glance, fit the currently accepted model for migration.

This release of a light oil appears to be in disagreement with the well-documented fact that source strata rich in terrestrial-derived organic matter generate heavy, waxey crude oils (Hedberg, 1968).

an early stage of expulsion, controlled by shale compaction and associated chromatographic [continues]

## Page 18 (David's p. 8)

separation effect, appears to precede the main phase of primary migration (oil-phase expulsion, responsible for formation of major oil accumulations).

Summary (See Copy)

[9] DK-1/76-3(3/92) p 403-406: The Quantification of Diffusive HC losses Through Cap Rocks of Natural Gas Reservoirs - A Reevaluation: Krooss, Leythaeuser, Schaefer

On a geologic time scale, natural gas accumulations must be considered as dynamic systems (Leythaeuser et al, 1982). Their survival & evolution depend essentially on the ratio of gas supply from underlying source rocks & gas losses through the cap rock, & its variation with time.

Diffusive transport through the water saturated cap rock probably is the least efficient of the 3 leakage mechanisms

Conclusion: The recalculation of gas losses from the Harlingen gas field yielded half-life times substantially larger (70 MY vs. 4.5 MY) than in the original computations by Leythaeuser et al (1982), indicating that diffusive losses have been overestimated for this particular case history. Diffusion losses, however, should not be generally disregarded as adverse effects with respect to the persistence of gas accumulations with time because many reservoirs have significantly smaller cap rock thicknesses than Harlingen field.

[marginalia: red "US.WS WRONG ASSUMPTIONS AGAIN" at left]

DN-1/76-11(11/92) p 1839-1841: Discussion by Nelson & Simmons.

\* Nelson - showed that the porosity effect was already accounted for in the radical cross-section and area over which diffusion can occur and stated.

"Assuming a cap rock temperature of 70°C & a pore-water salinity of 3.6 wt %, the effective diffusion coefficient of methane should be approx. 5.5 x 10⁻¹⁰ m²/sec. Adding this change to that of the porosity correction yields a [continues]

## Page 19 (David's p. 9)

t₁/₂(out) of only 4.4 M.Y. surprisingly close to the original first-order approx. of 4.5 M.Y. by Leythaeuser et al (1982).

As noted by Pandey et al (1972) and Leythaeuser et al (1982), diffusion of light-molecular-weight molecules through cap rocks is a significant factor in the longevity of natural gas fields. Relatively large volumes of natural gas even through thick cap rocks, may occur in only a few million years.

DK-2/76-11 (Nov 1992) p1842-1846: Reply by Krooss

[10] Effusive Microseepage: A First Approximation Model for Light HC Movement in the Subsurface: Gerald K. Arp (1992) [marginalia: APGE]

However, surface geochemistry case studies show measurable vertical microseepage at the surface over many fields, and some studies indicate changes in the reservoirs can be detected at the surface by geochemical techniques in as little as 20 years.

\* Actually, we now know this can occur in as short as one year.

Arguments Against microseepage through shales.

3 MAIN Reasons 1-) the pore sizes typically found in shales are too small (<3nm) to allow transmission of any but the very smallest HC molecules 2) small pores develop high differential capillary pressures (to 40 atm) which must be overcome if fluids are to move through them; and 3) shales have very low permeability (10⁻⁵ to 10⁻⁸) Darcies, which inhibits the movement of molecules small enough to pass through the pores (Hunt 1979, Levorsen 1958, Price 1985, 1986)

Possible Mechanisms for Microseepage

1) Diffusion, 2) Effusion, 3) Movement of dissolved HC in formation waters, & 4) microbubbles

Relationship of Permeability to Velocity

The calculations for Case 1 & Case 2 yield a velocity of V = 2.17 x 10⁻² cm/sec, or 2.25 x 10⁴ ft/YR (22500 ft/YR) as calculated the velocity for V is faster than velocities [continues]

## Page 20 (David's p. 10)

determined from field evidence,

we know from the gas storage literature that it is possible, to quickly move large amounts of light HC's from a normally sealed reservoir rock upward through normally bedded unfaulted sediments. Araktingi et al (1982) have shown that gases placed in the Leroy natural gas storage reservoir can escape to the surface in less than 5 years if the unit is overpressured during gas storage operations. Migration rates of 11000 feet per year can be calculated for this case. Hunt (1979) had noted similar rates for gas storage units in the Soviet Union.

Similar rates of movement can be shown for disrupted gas caps over stratigraphic fields. Arp (1992) found that gases escaping from a gas cap at the Patrick Draw field reached the surface in 10 to 20 years with a conservatively estimated velocity of 250 ft/year. Velocities of 250 to 9000 ft/yr are consistent with earlier published rates of 250 ft/yr by Horvitz (1969) for the Hastings field. For this study an approximate upper limit velocity of 250 ft/yr or 2.42 x 10⁻⁴ cm/sec, from reservoir to the surface, will be used.

[11] MPG 1993 Vol. 10: Diffusion Model For Predicting Reservoir Gas Losses: F. Montel, G. Caillet & A. Pucheu

The numerical model aims to predict gas losses over geologic time and the distribution of free gas within the caprock and the overlying sediment!

HC accumulation at the present time are: gas cap = 70 x 10⁹ m³; dissolved gas = 30 x 10⁹ m³; and oil = 200 x 10⁶ m³. [red: 70 BCM = 2.47 TCF] [red: 30 BCM = 1.06 TCF]

Case History #1: No gas migration pathway through the caprock.

→ The cum. reservoir losses are 220 m³/m² or 15.4 x 10⁹ m³ for the whole structure. [red: 15,4 BCM; 721,7 CF/ft²; 543.8 BCF]

Case History #2: migration pathway through the Cretaceous calcareous layers.

The gas flux between the calcareous layers and the reservoir was downwards & the cum. flux is 137 m³/m² or 9.6 x 10⁹ m³ for the whole structure; the reservoir is filled with gas from the caprock!! [red: 4994 CF/ft²]

Above the Cretaceous structure the cum. flux (upward) is 294 m³/m² (20.6 x 10⁹ m³) [red: 20.6 BCM = 727.4 BCF] [continues]

## Page 21 (David's p. 11)

[12] 79-3 (2/95) 470-430 [DLK-1]: Molecular Nitrogen in Natural Gas Accumulations: Generation from Sedimentary Organic Matter at High Temperatures: Litke, Krooss, Idiz & Frielingdorf

Abstract (MAIN CONCLUSIONS)

This concept predicts that Nitrogen is generated from organic matter at later stages of maturation than methane.

Data on the total methane & nitrogen generation from highly mature coal indeed prove that nitrogen generation exceeds methane generation at high temperatures; that is, highly mature coals, during laboratory pyrolysis, produce more nitrogen than methane.

Clay-rich rocks types, such as shales & mudstones, are known to contain greater quantities of nitrogen than other sedimentary rocks except coals. Our data indicate that much of this nitrogen is fixed in dispersed organic matter and that little nitrogen occurs in silicates.

[13] [DN-2] 79-7 (7/95) 1064-1074: Diffusion of Methane & Ethane Through the Reservoir Caprock: Implications for the Timing & Duration of Catagenesis: Nelson & Simmons

To maintain natural gas reserves for even short periods of geologic time, there must be an influx of gas into the reservoir to balance any loss.

The minimum influx rate necessary to maintain the reserves since is the same as the diffusive loss rate.

McClave Field with 5% Φ Caprock: entire volume of methane would have to be replaced in 2.3 MY and 5.3 MY for ethane. Using 10% Φ the replacement time falls to < 500,000 years for methane & 4.2 MY for ethane. [red: 57 BCF]

∴ there must be ongoing generation and migration of gas to the reservoir to at least balance the diffusional loss. This balance requires that catagenesis be an ongoing and recent phenomenon.

\* - No reservoir caprock or trap has ever been shown to be a perfect seal to HC migration.

## Page 22 (David's p. 12)

[13 Cont]

Faults, fractures, microfractures and pore space in the reservoir caprock precludes a perfect seal. The caprock or trap only retards migration, allowing the temporary accumulation of reserves within the reservoir.

Molecular diffusion, generally regarded as the slowest loss mechanism, is always present if a concentration gradient (chemical potential gradient) exists (Nernst, 1923; Bockris & Reddy, 1970).

\* - For reserves to remain constant for even short periods of geologic time, there must be an influx to the reservoir to balance any loss, otherwise the reserves must decline over time.

The net effect of all of the conservative choices in the calculations is to render diffusive fluxes that may be over an order of magnitude too low, and replacement times that are an order of magnitude too high. a caprock porosity of 10% instead of 5% gives a five-fold increase in the flux.

57 BCF of gas (40 bcf of methane) must be generated & migrated every 2.21 MY

40 BCF = 40,000,000,000 CF / 2,210,000 YR = 18,100 CF/YR

= 18.1 MCF/YR x 4500 Years = 36,450 MCF in 4500 YRS

Diffusion calculations indicate that natural gas fields are not static environments into which natural gas migrated millions of years ago to remain trapped until discovery. In reality the oil and gas field environment is an active, dynamic system in a constant state of change.

In the real world, diffusion may be occurring at a much faster pace.

[13a] [DW-1] 80-9(9/96) 1483-1485: Discussion: H.H. Wilson

\* Wilson's Main Objection "To satisfy this conclusion one must accept long-duration catagenic generation of gas & long-distance gas migration over an extended time span."

He quotes good examples of oil fields above gas fields that he indicates have not communicated. [continues]

## Page 23 (David's p. 13)

[13a]

giant Wattenberg Cretaceous, 2) supergiant Hugoton gas field, 3) Paleozoic reservoirs in Appalachian Basin, 4) Khuff gas reservoir under Ghawar, 5) IN Qatar, the Idd el Shargi dome, offshore and the Dukhan anticline onshore & the Qatar North Field.

"The consequences that would logically stem from this assumption are that oil & gas will be found in traps regardless of their time of formation; however, in the real world of oil & gas distribution this just does not happen, and the empirical evidence for accumulation only in early traps is overwhelming (Wilson 1975, 1982, 1990)."

[marginalia: red "Wilson Is correct in this point but he doesn't consider A young earth" at left]

\* "If the concept of methane leakage & catagenic replacement so confidently put forward by Nelson and Simmons is to be taken seriously, it must address the logical consequences & their bearing upon gas accumulations around the world."

[13b] DN-3 80-9(9/96), 1486-1489: Reply by Nelson & Simmons

\* Their reply "In the 140 years following the 1855 publication of Ficks 1st law of steady-state diffusion, no exception to it has been found, despite efforts to do so. At its simplest, Ficks law states that flux is proportional to a gradient, a proposition that applies to all four transport equations (diffusion, thermal conduction, viscous flow & electrical conductivity; e.g. Atkins 1986; Levine 1988). Any geologic concept that violates the fundamental laws of chemistry & physics cannot be valid.

The striking similarity of Wattenberg and Spindletop gases (table 1) does not rule out diffusion or other leakage from Wattenberg, and may be considered evidence of vertical migration.

In regards to Florence field he quotes DeFord (p87) "A scheme like the foregoing tacitly postulates that the accumulation of blood in the fissures - the commercial accumulation - was very recent, as recent as the present topography."

## Page 24 (David's p. 14)

[13b Cont]

In regards to Hugoton, Nelson says "Wilson's example provided chemical evidence for migration."

Our calculated methane flux at McClave has a volume of 3.86 x 10⁻³ liter/m²/YR or only 1/100,000 liter/m²/YR. In any agricultural area, surface and near-surface biologic production of HCs could swamp the signal of the minute volume rising from depth, making an observation almost impossible.

1/100,000 liter/m²/YR x 4500 YRS x 365 days = 16,431 liters/m² / 4500 YRS

" " " x 2.21 MYRS x 365 days = 8067 liters/m²/2.21 MYR

Ratio is 491 Times

Well-site geologists & mud loggers commonly observe an increase in flow-line background gas as the caprock above a productive reservoir is drilled. The observed increase in C₁-C₄ across the caprock is exactly what would be expected if diffusion were occurring.

Nelson said "Wilson claims "overwhelming" evidence for "accumulation only in early traps"" by which (we think) he means early generation of oil preferential entrapment in early formed traps, and the absence of later migration (Wilson 1990, 1996 p 1489)" Then Nelson gives the example Dinch u Bi-keyah as an argument against Wilsons claims. But this field is a very unique field. See summary of Wilsons papers for Volume numerous examples of early migration into early traps with barren late traps in close proximity.

\*\* Nelson & Wilson are at odds because neither will give up on deep time. \*\*

[13c] DK-3 81-1(1/97) 155-161: Discussion: Krooss & Leythaeuser

In summarizing the (Nelson & Simmons) article, Krooss said "N&S (1995) suggested that high turnover rates (less than 1 m.y. to several m.y.) due to diffusive losses are a characteristic feature of natural gas accumulations."

## Page 25 (David's p. 15)

[13c Cont]

and they consequently postulated continuous and rapid gas generation as a prerequisite for the persistence of natural gas reservoirs. However, by using a rather arbitrary set of parameters as the basis of their calculations, they depicted a scenario that must be considered as quite exceptional in terms of reservoir geometry and pay zone thickness. Their conclusions on the dynamics of natural gas accumulations are therefore questionable and must be considered with caution.

Krooss went on to say "The combination of an extremely small average pay zone thickness (according to the data presented), large areal extent and a relatively thin caprock represents a geological situation that is highly prone to rapid depletion by diffusive losses."

\*\* Note: Even if Krooss was correct, the existence of the McClave field for even a small geologic time (≈ 2 m.y.) has to be explained and as Nelson's reply will show McClave is actually the Norm.

[13d] [DN-4] 81-1(1/97) 162-167: Reply: Nelson & Simmons

In addressing "missed references" Nelson said "Regrettably, we did miss Montel et al (1993), a fundamentally different approach that supports our contention that our calculations are (too) conservative".

Nelson says of "(K&L 1997) presume without evidence, the McClave field results are invalid for most gas fields of SE Colorado. Table 1 presents data 18 SE Colo. fields including McClave. The play area represented exceeds 25000 km², Table 1 reveals that McClave field is the largest field & has the 5th thickest Cap Rock, the 5th highest % of CH4, the 4th highest % of C₂ HC, and the 2nd lowest percentage of N₂.

## Page 26 (David's p. 16)

[13d Cont]

A smaller field would have been dismissed as "arbitrary". McClave's cap rock, dismissed as "arbitrary" is 40% thicker than the area average.

Nelson says "McClave field is not one of 5% of the world's fields that contain over 95% of the reserves (Riva, 1983). It is one of the 95% of the 30000 + fields that are small. McClave field is ordinary & mundane." He contrasts McClave to the super giant Urengoy-type that was one of Krooss' "typical" fields. He goes on to say "If McClave field is unrepresentative, it is because it is too big, not too small."

Some Numbers From Table 2 [red: per MY YR 1 BCF/mMYR = 1 MCF/YR]

| Field | CapRock Ft | Area KM² | Reserves BCF | Tot Flux Loss (BCF) | CF/m²/mMYR | CF/m²/YR |

|---|---|---|---|---|---|---|

| Urengoy | 930 | 4200 | 230,000 | 24.0 | 5.7 | 5.7 x 10⁻⁶ |

| Urengoy \* | 930 | 4200 | 230,000 | 198.5 | 47.0 | 4.7 x 10⁻⁵ |

| North Sea | 120 | 70 | 2,474 | 27.2 | 389.0 | 3.89 x 10⁻⁴ |

| Harlingen | 390 | 3.75 | 88 | 0.54 | 144.0 | 1.44 x 10⁻⁴ |

| McClave | 39.6 | 135 | 57 | 18.4 | 136.0 | 1.36 x 10⁻⁴ |

\* Calculated using Krooss & Leythaeuser's (1967) All others use Nelson & Simmons 1995

→ The "geological situation that is highly prone to rapid depletion by diffusive losses" (Krooss & Leythaeuser 1997, p 158) is also quite common, and may be the rule rather than the exception. McClave field is more typical of the average gas field than the supergiant Urengoy field, a hypothetical giant, or the Harlingen field. A minimum loss of 600 BCF or 4.9 TCF of Urengoy represents the loss of a large a giant gas field or numerous smaller (122 to 1000 Nee Nashes) in 25 M.Y. by diffusion alone. "How large a field or how many fields must be lost before Krooss & Leythaeuser (1997) consider it significant."

\*\*→ Whether by diffusion alone or with additional faster loss mechanisms, small fields and giants alike have been lost. Diffusion is but the slowest loss mechanism, and is always present \*\*

## Page 27 (David's p. 17)

[14a] 1996 AAPG Mem #66: Chapter 1: "Distribution of Subsurface HC Seepage in Near-Surface Marine Sediments" p 1-14. Michael Abrams

Transient Nature of Leakage

→ Surface HC seeps & anomalies have been shown to appear & disappear in relatively short times (weeks to months). Sivabdorvon (1974) reported that a surface geochemical anomaly increased in intensity over the Illes field in Bastrop County, Tx, during repressuring for secondary recovery studies. Coleman et al (1977) demonstrated leakage of gas from 1000m-deep underground storage reservoirs into shallow-water wells within a 1 year period. Horvitz (1985b) collected surface sediment cores in 1946 over the Hastings field in Brazoria County, Texas, and the halo pattern of anomalous hydrocarbon values outlined the producing area. Then in 1968, Horvitz recollected shallow-sediment samples after the field was depleted. The 1968 survey showed no significant pattern of anomalous HC values. All these observations clearly indicate that seepage & migration are dynamic (ongoing).

[14b] 1996 AAPG Mem #66: Chapter 11: "Migration - A View from the Top". p139-155. Martin D. Matthews [marginalia: Copy Fig 3]

→ "Perfect seals, which do not leak at all, are the type of seals most petroleum geologists visualize, but they rarely occur."

→ "The majority of HC molecules are larger than the size of small shale pores & approximately equal to the median pore size of shales."

[marginalia: Copy Fig 4]

→ "Note that shale compaction results in a logarithmic decrease in K down to a depth of about 3.5km." = 11484'

Rate of Transport

→ "Arp (1992) summarized estimates of vertical seepage velocities of 75-300 m/year. Volume rate estimates have been made in the marine environment by collecting bubbles. The avg. oil seep flow rate compiled by Clarke & Cleverly (1991) is 50m³ (300 bbl) per year. These rates clearly indicate separate-phase migration along narrow migration pathways. Diffusion rates are much slower but operate over much larger areas."

[red marginalia: 24 to 984 ft/YR; IF 50m³ = 300 Bbl then 1 m³ = 6 Bbl]

## Page 28 (David's p. 18)

[14b Cont]

Conclusions

→ "Most reservoired HC's have compositions that are controlled by the characteristics of their source rock, not the process of their transportation. Diffusion of light HC's into cap rocks explains the abundance of reservoirs undersaturated with light gases."

→ "Transport of HC's along a kerogen network is limited to rocks with TOC contents over about 1%. These constitute a small percentage of most migration paths. It may be an important expulsion mechanism within source rocks, but it would seem to render these source rocks relatively ineffective as a top seal. Since many source rocks do act as top seals, the mechanism is thought to be an exception."

Postexpulsion Model

=> The author states "Second, within the carrier bed buoyancy (assisted by gas expansion) is a major control on migration. Water velocities generated by compaction are generally insignificant compared to buoyancy-driven velocities and as a first approximation can be ignored."

Note: He comes to this conclusion because of his belief in deep time. There is abundant evidence in the sedimentary column of high velocities of water escape generated by compaction.

=> He goes on to say "Third, the flow of HC's from a reservoir into a seal is dominated by the penetration of a separate phase. This is accomplished by buoyant pressure overcoming capillary restriction. Finally within the seal, the HC's move through discrete pathways of one or more continuous threads until they encounter a carrier bed."

=> And he says "The rates of leakage through the sequence of seals between the source rock and the earth's surface control the pattern of HC accumulations along the post-expulsion migration route & both the magnitude and spatial properties of surface seepage."

## Page 29 (photocopied figure page)

[red \*] Figure 3 — Comparative sizes of shale pores and molecules of hydrocarbon, water, and kerogen. [printed figure; log scale from 1 nanometer to 0.1 cm]

Bars plotted (left to right, approximate): Shale Small pores; Shale – Median Pores; Shale – Large Pores; Oil in Water Emulsions; Micelles – Median Diameters; Colloidal Oil; Bitumen in Oil; Aggregated Asphaltenes; Largest Asphaltenes; Aromatic; Normal Paraffins; Methane; Water; Kerogen Particles.

Printed text:

"their light ends and other properties. This suggests that the migration and entrapment process has some control over the chemical characteristics of reservoired hydrocarbons, but in most cases this only affects the detailed chemistry, not the overall structure."

"increasingly less likely as the traversed path lengthens due to the increased probability of a continuous large pore network terminating into a small pore throat. Indeed, even the flow of the comparatively small water molecule often requires significant pressure gradients to overcome the restrictions to flow common in shales."

\* Physical Sizes

"The majority of hydrocarbon molecules are larger than the size of small shale pores and approximately equal to the median pore size of shales (Figure 3). Methane molecules are slightly larger than the size of small shale pores [text cut off at page bottom]"

Pore Throat Heterogeneity and Interstitial Continuity [heading, text continues]

## Page 30 (photocopied figure page, David's p. 20)

[red \*] Figure 4 — Graph of permeability in Tertiary shales with depth. (After Neglia, 1979.) [printed scatter plot; y-axis Depth (km) 1–6; x-axis Permeability (darcys) 10⁻⁸ to 10⁻⁴, log scale; data points trend from ~10⁻⁶–3×10⁻⁶ darcys at 1–1.5 km down to ~10⁻⁸ darcys at 5–5.2 km]

"The relationship between net path permeability and frequency of occurrence of a given path is summarized in Figure 6. The net path permeability of a given pore network is determined by the smallest pore throat in that [text cut off at page bottom]"

## Page 31 (David's p. 21) — VIII-C bibliography

VIII) C)

Diffusion of Fluids Through Porous Media With Implications in Petroleum Geology: G.N. Pandey, M. Rosin Tek & Donald L. Katz [red: 1974]

Diffusion of Light Hydrocarbons Through Near-Surface Rocks: D. Leythaeuser, R.G. Schaefer

Near-Surface Evidence of Migration of Natural Gas From Deep Reservoirs & Source Rocks: W. Stahl, E. Faber

DL-1 66-4 (4/82) 408-429 4) Role of Diffusion in Primary Migration of Hydrocarbons: Leythaeuser, Schaefer, Yokler [red: 4/82]

DL-2 67-6 (6/83) 889-895 5) Diffusion of Light Hydrocarbons in Subsurface Sedimentary Rocks: Leythaeuser, Schaefer, Pooch [red: 6/83]

67-6 (6/83) 932-952 6) Predictions of Oil or Gas Potential by Near-Surface Geochemistry: V.T. Jones & R.J. Drozd [red: 6/83]

JPT 1984 7) Leroy Storage Facility, Uinta County, Wyoming: A Case History of Attempted Gas-Migration Control: R.E. Araktini, M.E. Benefield, Z. Bess enyei

DL-3 68-2 (2/84) 196-219 8) A Novel Approach for Recognition & Quantification of Hydrocarbon Migration Effects in Sand-Shale Sequences: Leythaeuser, MacKenzie, Schaefer, Bjoroy [red: 2/84]

DK-1 76-3 (3/92) 9) The Quantification of Diffusive Hydrocarbon Losses Through Cap Rocks of Natural Gas Reservoirs - A Reevaluation: B.M. Krooss, Leythaeuser, Schafer [red: 3/92]

APGE Bull (12/92) 10) Effusive Microseepage: A First Approximation Model for Light Hydrocarbon Movement in the Subsurface: Gerald K. Arp [red: 12/92]

MPG Vol 10 1993 11) Diffusion Model for Reservoir Gas Losses: F. Montel et al [red: 1/92]

DLK-1 79-3 (3/95) 470-436 12) Molecular Nitrogen in Natural Gas Accumulations: Generation From Sedimentary Organic Matter at High Temperatures: R. Litke, B Krooss, E. Idiz & J. Frielingsdorf

## Page 32 (David's p. 22) — VIII-C bibliography, continued

DN-2 79-7 (7/95) 1064-74 13) Diffusion of Methane and Ethane Through the Reservoir Cap Rock: Implications For the Timing and Duration of Catagenesis: Jon S Nelson & E.C. Simmons [red: 7/95]

AAPG Mem 66 (1996):

Chap 1 14) a) Distribution of Subsurface Hydrocarbon Seepage in Near-Surface Marine Sediments: Michael A. Abrams [red: 1996]

Mem 66 Chap 11 14) b) Migration - A View from the Top: Martin D. Matthews [red: 1996]

Mem 66 Chap 12 14) c) Comparison of Light Hydrocarbon Microseepage Mechanisms: Ronald W. Klusman, M.A. Saeed [red: 1996]

Mem 66 Chap 13 14) d) Comparison of Seepage & Seal Leakage Rates: Clayton & Dando [red: 1994]

Chap 14 14) e) Molecular Diffusion of Light Hydrocarbons in Sedimentary Rocks and Its Role in Migration & Dissipation of Natural Gas: Krooss & Leythaeuser

Chap 16 14) f) Hydrocarbon Flux Variations in Natural & Anthropogenic Seeps: Victor T Jones III, Stephen G. Burtell [red: 1996]

15)

83-1 (1/99) 170-185 16) Model For HC Microseepage & Related Near-Surface Alterations: Saunders, Burson, Thompson

17)

84-11 (11/2000) 1775-1789 18) Evaluation of Possible Gas Microseepage Mechanisms: Atton Brown [red: 11/2000]